Computer Networks

ASSIGNMENT 1

Make the Data of a Smart Manufacturing Plant Arrive Correctly and On Time

| Name | R. Vishwa |
| --- | --- |
| Register Number | 1911260146 |
| Course code | CSA07 – Computer Networks |
| Nature | Individual – Total marks 50 |
| GitHub repository | CSA07-1911260146-Assignment1 – link: https://github.com/Rvishwa28/CSA0707-COMPUTER-NETWORK.git |
| Python version used | Python 3 (confirm installed version after running the programs) |

# Task 1 – Size the Traffic Before Choosing Anything

## 1.1 Total traffic

| Source | Working | Traffic |
| --- | --- | --- |
| Robot controllers | 40 × (200 B × 8 bits) ÷ 2 ms = 40 × 0.8 Mbps | 32.00 Mbps |
| Vibration sensors | 600 × 50 B/s × 8 = 240,000 bit/s | 0.24 Mbps |
| CCTV cameras | 24 × 4 Mbps (continuous) | 96.00 Mbps |
| Office users | 150 × 10 % = 15 active × 2 Mbps | 30.00 Mbps |
| Total, one line | 32 + 0.24 + 96 + 30 | 158.24 Mbps |
| Plant, 8 lines | 8 × 158.24 | 1,265.92 Mbps (≈ 1.266 Gbps) |

## 1.2 Most data vs strictest delay

The CCTV cameras send the most data (96 Mbps = 60.7 % of the line’s traffic). The robot controllers have the strictest delay requirement (commands within 10 ms) even though they send only 32 Mbps. They are not the same source – volume and urgency are different properties, so a design sized only on bandwidth would ignore the robots.

## 1.3 Is a single 1 Gbps uplink enough?

- One line: 158.24 Mbps = 15.8 % of 1 Gbps – enough.

- Whole plant (8 lines): 1,265.92 Mbps = 126.6 % of 1 Gbps – not enough; the uplink is overloaded by 265.92 Mbps.

- Shared queue with CCTV: if robot commands wait in the same FIFO queue as CCTV, each command may sit behind a burst of camera frames. A single 1,500-byte frame takes 12 µs to send, but a filled 1 MB buffer takes 1 MB × 8 ÷ 1 Gbps = 8 ms to drain – 80 % of the robots’ 10 ms budget in one queue. Once the offered load exceeds 1 Gbps the buffer overflows and commands are dropped. Fix: separate VLAN and strict-priority queue (802.1p / DSCP EF) for robot traffic, and a 10 Gbps uplink.

## 1.4 Ranking by harm of a 100 ms delay

| Rank | Traffic | Reason |
| --- | --- | --- |
| 1 (worst) | Robot commands | 100 ms is 10× the 10 ms deadline; robots act on stale commands – collisions, scrap parts or a safety stop. |
| 2 | CCTV | Live view lags and jitter breaks video playback; frames are lost if buffers are small – but recording is not safety-critical. |
| 3 | Office / ERP / e-mail | Interactive use notices 100 ms but remains usable; TCP absorbs it. |
| 4 (least) | Vibration sensors | Report once per second, so 100 ms is only 10 % of the reporting period; trend data, not control. |

## Alternative Concept Analysis – motion-triggered CCTV (5 % of the time)

- CCTV traffic = 96 × 0.05 = 4.8 Mbps (saves 91.2 Mbps per line, 95 %).

- One line = 32 + 0.24 + 4.8 + 30 = 67.04 Mbps; 8 lines = 536.32 Mbps = 53.6 % of 1 Gbps.

- Decision: on average the 1 Gbps uplink is no longer the bottleneck. Remaining risk is queueing delay when many cameras trigger together (all 24 triggered = back to 96 Mbps per line) – still handled by priority queues. Cost: cameras need local storage, and live viewing/forensic delay increase.

complete calculations with units, delay-versus-volume comparison and a justified ranking.

# Task 2 – Count the Domains in the Contractor’s Drawing

## 2.1 Domain counts

| Part of the network | Collision domains | Why |
| --- | --- | --- |
| Hub 1, Hub 2, Hub 3 (8 PCs each + uplink to switch) | 3 | A hub repeats every signal to all ports, so each hub with its 8 PCs and its switch port is one shared segment. |
| Switch 1 ↔ router G0/0 | 1 | One link between two devices. |
| Router G0/1 ↔ Switch 2 | 1 | One link between two devices. |
| 12 IP phones on Switch 2 | 12 | Each switch port is its own collision domain. |
| Total | 17 | 3 + 1 + 1 + 12 |

Broadcast domains = 2. Switches and hubs forward broadcasts; the router does not. Interface G0/0 (switch 1, the three hubs and 24 PCs) is one broadcast domain and G0/1 (switch 2 and 12 phones) is the other.

## 2.2 OSI layer of each device and why a hub cannot stop collisions

| Device | OSI layer | What it does |
| --- | --- | --- |
| Hub | 1 – Physical | Regenerates and repeats electrical bits to every port; does not read addresses. |
| Switch | 2 – Data link | Reads the destination MAC, learns addresses and forwards only to the right port. |
| Router | 3 – Network | Reads IP addresses, routes between networks and blocks broadcasts. |
| PC / IP phone | All layers (7 down to 1) | End systems that run applications. |

A hub works only at layer 1: it has no memory of frames or MAC addresses, so it cannot buffer or separate traffic. Two PCs sending at the same time put overlapping signals on the shared wire – a collision – and the hub passes that on to every port. (Shared Ethernet therefore needs CSMA/CD.)

## 2.3 Point-to-point vs multipoint

- Point-to-point (two devices only): Switch 1 ↔ router G0/0; router G0/1 ↔ Switch 2; each IP phone ↔ Switch 2 port; each hub ↔ switch uplink as a cable.

- Multipoint (shared by many devices): each hub segment – 8 PCs plus the uplink share one medium.

## 2.4 Standards and bodies

| Technology | Standard | Body |
| --- | --- | --- |
| Ethernet | IEEE 802.3 | IEEE |
| Wi-Fi | IEEE 802.11 | IEEE |
| IPv4 | RFC 791 | IETF |
| OSI model | ISO 7498 | ISO |
| 5G | 3GPP specifications (IMT-2020) | 3GPP / ITU |

Why buy only standard-compliant equipment: devices from different vendors interoperate; the plant is not locked to one supplier; equipment can be replaced or extended later (the network will grow to 5 plants); and conformance testing/certification (e.g. Wi-Fi Alliance, Ethernet) reduces risk.

## Alternative Concept Analysis – replace the three hubs with three unmanaged switches

|  | With hubs | With 3 switches |
| --- | --- | --- |
| Collision domains | 17 | 41 (24 PC links + 3 switch uplinks + 1 + 1 + 12 phones) |
| Broadcast domains | 2 | 2 (unchanged) |

What it fixes: collisions disappear (every link is full-duplex and CSMA/CD is not needed); each PC gets its own 100 Mbps/1 Gbps instead of sharing a segment.

What it does not fix: the 24 PCs are still in one broadcast domain; unmanaged switches have no VLANs, QoS, security or monitoring; no layer-3 separation; and without spanning tree, a mistaken loop can bring the network down.

correct domain counts on a marked drawing, device-layer reasoning and a standards table.

# Task 3 – Read a Captured Frame Layer by Layer

## 3.1 Layer and PDU of each line

| Line of capture | OSI layer | PDU |
| --- | --- | --- |
| Ethernet II (MACs, Type 0x0800) | 2 – Data link | Frame |
| IPv4 (10.10.1.25 → 10.10.9.4, TTL 61, Protocol 6) | 3 – Network | Packet |
| TCP (port 51522 → 502, Seq 1201, Ack 88) | 4 – Transport | Segment |
| Data (12-byte Modbus write-register command) | 7 – Application (5–7) | Data / message |

## 3.2 Routers crossed

The controller set TTL = 64; the packet shows TTL = 61. Each router subtracts 1, so 64 − 61 = 3 routers were crossed before the capture point.

## 3.3 Bytes on the wire and overhead

|  | 12-byte command | 1,400-byte camera packet |
| --- | --- | --- |
| Data | 12 | 1,400 |
| TCP header | 20 | 20 |
| IPv4 header | 20 | 20 |
| Ethernet header (6 + 6 + 2) | 14 | 14 |
| FCS | 4 | 4 |
| Total on the wire | 70 bytes | 1,458 bytes |
| Overhead | 58 B = 82.86 % | 58 B = 3.98 % |

## 3.4 Which layer handles each fault

| Fault | Layer | What happens |
| --- | --- | --- |
| (a) MAC address wrong | 2 – Data link | NIC/switch does not accept the frame; it is dropped. |
| (b) TTL reaches 0 | 3 – Network | The router discards the packet and returns ICMP “Time Exceeded”. |
| (c) Destination port closed | 4 – Transport | The host replies with a TCP RST (connection refused). |

## 3.5 TCP/IP model mapped onto the seven layers

| OSI layers | TCP/IP layer |
| --- | --- |
| 7 Application, 6 Presentation, 5 Session | Application |
| 4 Transport | Transport |
| 3 Network | Internet |
| 2 Data link, 1 Physical | Link (Network access) |

## 3.6 Complete the Code – Task03/layers.py

def send(message):

pdu = message

for layer, tag in LAYERS:

head = "[" + tag + "|"

tail = "|FCS]" if tag == "ETH" else "]"

pdu = head + pdu + tail

print(f"{layer:<13}adds {tag:<4} -> {len(pdu):>3} bytes")

return pdu

1,400-byte message: the toy headers add 28 bytes in both cases, but 28 ÷ 40 = 70 % overhead for the 12-byte message against 28 ÷ 1,428 = 1.96 % for 1,400 bytes – the same lesson as the real capture.

## Alternative Concept Analysis – command directly in an Ethernet frame (EtherType 0x8892)

|  | Modbus/TCP/IP/Ethernet | Direct in Ethernet (0x8892) |
| --- | --- | --- |
| Bytes per command (nominal) | 12 + 20 + 20 + 14 + 4 = 70 | 12 + 14 + 4 = 30 |
| Bytes saved | – | 40 bytes (57 %) |
| Commands per second on 100 Mbps | 100 Mbps ÷ (70 × 8) = 178,571 | 100 Mbps ÷ (30 × 8) = 416,667 |
| Extra commands freed | – | ≈ 238,096 per second |

Practical note: Ethernet’s minimum frame is 64 bytes, so a 30-byte frame is padded to 64; the real saving is then 70 → 64 bytes (6 bytes, 195,312 commands/s). The main gain is lower processing time, not bandwidth.

What the plant loses by giving up IP: routing – frames cannot leave the local layer-2 network, so no link to other lines/plants or the head office; no TTL and no IP firewalling/addressing; no TCP – so no ordering, retransmission or flow control (the application protocol must provide reliability itself); and the traffic is limited to one broadcast domain.

A layer-by-layer reading of the capture, overhead calculations (70 B/82.86 % and 1,458 B/3.98 %) and a fault-to-layer map.

# Task 4 – Choose a Medium for Each Run

## 4.1 Medium for each run

| Run | Chosen medium | Deciding reason |
| --- | --- | --- |
| (a) Robot cell next to arc-welding, 85 m | Cat6A S/FTP (shielded) – or multimode fibre if the noise is severe | 85 m is inside the 100 m twisted-pair limit; shielding resists welding EMI. Fibre is completely immune to EMI. |
| (b) Office floor → shop-floor switch, 140 m | Multimode fibre (OM3/OM4) | The trap: 140 m looks short, but is over the 100 m limit of any twisted-pair Ethernet cable. |
| (c) Shop floor → data centre, 550 m | Single-mode fibre (OS2) | 550 m is the very edge of 1000BASE-SX on multimode and beyond 10G multimode reach (OM4 ≈ 400 m); single-mode covers up to 10 km with room to grow. |
| (d) Plant → head office, 400 km | Leased line (carrier fibre) or carrier WAN service | No private cable can be run 400 km; the carrier’s long-haul fibre with amplifiers provides an SLA. |

Which run cannot use copper Ethernet at all: (d) 400 km – no copper Ethernet exists at that distance (attenuation; Ethernet copper is limited to 100 m). Runs (b) 140 m and (c) 550 m also exceed the 100 m limit, so copper is out for them too unless an extra switch is inserted; the trap is (b).

## 4.2 Shannon capacity of a 250 MHz cable

| SNR | Working | Capacity |
| --- | --- | --- |
| 40 dB (normal) | 250 MHz × log₂(1 + 10 000) = 250 × 13.288 | 3,321.96 Mbps |
| 15 dB (near welders) | 250 MHz × log₂(1 + 31.62) = 250 × 5.028 | 1,256.95 Mbps |

Why measured speed is always below the Shannon limit: Shannon is a theoretical upper bound that needs ideal coding and infinite complexity; real links use finite modulation and coding, non-ideal noise (not white), headers, preamble/inter-frame gaps, protocol overhead, retransmissions and equipment losses.

## 4.3 Complete the Code – Task04/media_limits.py

def nyquist(bandwidth_hz, levels):

return 2 * bandwidth_hz * math.log2(levels)

def shannon(bandwidth_hz, snr_db):

snr = 10 ** (snr_db / 10)

return bandwidth_hz * math.log2(1 + snr)

## 4.4 SNR needed for 56 kbps on a 3,100 Hz line

C = B log₂(1 + S/N) → S/N = 2^(56,000 ÷ 3,100) − 1 = 2^18.06 − 1 = 274,132 → 54.4 dB. A real analogue telephone line gives about 30–35 dB, which limits it to roughly 33.6 kbps. 54.4 dB is not realistic on an analogue line; 56k modems worked only because the telephone company’s end was digital (no analogue-to-digital quantisation noise).

## Alternative Concept Analysis – Wi-Fi 6 for the robot cell

160 MHz × log₂(1 + 31.62) = 804.45 Mbps at 15 dB – enough bandwidth for the 32 Mbps of robot traffic. But a wireless link is not acceptable for 10 ms robot commands: Wi-Fi uses CSMA/CA contention, so latency is not guaranteed; welding interference lowers the SNR randomly and causes retries; and delay/jitter can reach tens of milliseconds. Capacity is not the issue – determinism is. Keep shielded wired links for the robots; use Wi-Fi only for non-critical devices.

A justified medium for each run, trapped run (b) identified, and Shannon calculations with interpretation.

# Task 5 – Wi-Fi 6 or 5G for Moving Equipment?

## 5.1 Hand-overs per trip

|  | Wi-Fi 6 | Private 5G |
| --- | --- | --- |
| Coverage per access point/cell | 30 m | 300 m |
| Hand-overs on the 1.2 km route | 1,200 ÷ 30 = 40 | 1,200 ÷ 300 = 4 |
| Time per hand-over | 150 ms | 20 ms |
| Unreachable time per trip (600 s at 2 m/s) | 40 × 150 ms = 6.0 s (1.0 % of trip) | 4 × 20 ms = 80 ms (0.013 %) |
| One hand-over every | 15 s | 150 s |
| 2-ms commands lost per hand-over | 75 | 10 |

## 5.2 5G service types

| Traffic | 5G service type | Why |
| --- | --- | --- |
| AGVs (10 ms commands) | URLLC – ultra-reliable low-latency | Hard deadline and reliability. |
| CCTV cameras (96 Mbps per line) | eMBB – enhanced mobile broadband | High bandwidth video. |
| Vibration/temperature sensors (600 per line) | mMTC – massive machine-type communication | Many small, low-power devices. |

## 5.3 Where Bluetooth, Zigbee or LoRa beat Wi-Fi/5G

- Bluetooth (BLE): short range – tool/handheld pairing, wearables, asset tags; very low power.

- Zigbee: low-power mesh for battery sensors, lighting and environment monitoring; many nodes at low cost.

- LoRa: kilometres of range and years on battery for slow sensors (temperature, tank levels, yard/outdoor assets).

None is suitable for robot or AGV control or video – data rates are too low and latency too high.

## 5.4 Could Wi-Fi 6 with fast roaming (802.11r) meet the deadline?

802.11r cuts hand-over time from about 150 ms to roughly 50 ms or less (typical vendor figures), but even 50 ms is far above the 10 ms deadline: the AGV is still unreachable for 40 × 50 ms = 2 s per trip (about 1,000 missed 2-ms commands). Remaining risks: contention/jitter on a shared channel, interference from welding, AP overload and coverage holes – and 40 hand-over events per trip. 5G with only 4 hand-overs per trip is the better choice.

## Alternative Concept Analysis – LoRaWAN for the sensors

600 sensors × 50 B/s × 8 = 240 kbps, against a LoRa gateway capacity of about 5 kbps → 48 gateways needed (one gateway carries only 5,000 ÷ 400 = 12.5 such sensors). It does not fit. Decision: keep the 50 B/s vibration sensors on Wi-Fi (or wired); use LoRa only for slow sensors – e.g. 50 B every 60 s = 6.7 bps each, so one gateway serves about 750 sensors.

hand-over calculations, mapping of traffic to 5G service types and a justified wireless decision.

# Task 6 – Interpret an Ethernet Port Report

## 6.1 Meaning of the errors

- Late collision: a collision detected after the first 512 bits (64 bytes) of a frame were sent. On a correct half-duplex link collisions happen in that first 64-byte window; a late one means the other end transmitted while this end was already well into its frame – the other end is not following CSMA/CD (or the cable is too long).

- CRC error: the frame check sequence does not match – the frame was damaged; fragments cut short by late collisions are typical.

- Combination: the two ends disagree about duplex. The PC is forced to half-duplex (auto-negotiation off) while the gigabit switch port is operating full-duplex, so the switch transmits whenever it wants (no carrier sense) → the PC sees late collisions, CRC and receive errors; the switch sees runt/CRC errors. Result: very slow, retransmitted traffic.

## 6.2 Likely cause and exact fix

Cause: duplex mismatch – PC forced to 100 Mb/s half-duplex with auto-negotiation off, switch port set to auto/full.

Linux: sudo ethtool -s eth0 autoneg on (check with ethtool eth0; to force: sudo ethtool -s eth0 speed 100 duplex full autoneg off – then the switch port must be forced the same). Windows: Device Manager → Network adapter → Properties → Advanced → *Speed & Duplex* → Auto Negotiation (or PowerShell: Set-NetAdapterAdvancedProperty -Name "Ethernet" -DisplayName "Speed & Duplex" -DisplayValue "Auto Negotiation"). Best practice: auto-negotiation on at both ends (it is required for gigabit).

## 6.3 Addresses

- 01:00:5e:0a:01:02 – the first octet 0x01 has its least-significant bit (I/G) = 1, so it is a group (multicast) address, not a single device. The prefix 01:00:5e is the IPv4 multicast range; the low 23 bits map to a 224–239.10.1.2 group.

- 3c:52:82:4e:11:07 – the first three bytes 3c:52:82 are the OUI (organisationally unique identifier) – the NIC manufacturer’s IEEE-assigned ID; it is registered to Hewlett Packard. The last three bytes are the device-specific number.

## 6.4 Jumbo frames (9,000 bytes) for the CCTV storage server

Yes, they help: per-frame overhead (38 bytes including preamble and inter-frame gap) falls from 38 ÷ 1,538 = 2.47 % to 38 ÷ 9,038 = 0.42 %, and the server handles ~6 times fewer frames/interrupts. What must also change: the MTU must be raised on every device in the path – storage server, cameras/recorders, every switch port and VLAN end to end – otherwise frames are fragmented or dropped. Jumbo frames also stay inside one layer-2/MTU domain (not across the WAN).

## Alternative Concept Analysis – ring of industrial switches vs star

|  | Ring with fast ring recovery | Star of switches |
| --- | --- | --- |
| Recovery after one cable cut | Typically ≈ 10–50 ms (e.g. ERPS/MRP; classic RSTP 1–2 s) – traffic re-routes the other way round the ring | None – no alternate path; stays down until the cable is repaired (or needs a manual/redundant link) |
| Devices that lose connection | None after recovery (frames lost only during the brief switch-over) | The device(s) behind the cut cable – one device if an access link; all devices below a switch if an uplink |
| Cost | Fewer cables, but ring-capable (managed) switches | Simple, more cable to a central point |

(Recovery times are typical vendor figures for the protocols named.) For robots with a 10 ms deadline even a 30–50 ms recovery causes a few missed commands – so a ring is better than a star for availability, but robot cells should still fail safe.

reasoned interpretation of the report, corrective configuration, and correct answers on addressing and frame size.

# Task 7 – Will the Receiver Notice the Error?

## 7.1 CRC for data 1101011011, generator 1011

Append 3 zeros (degree of generator): 1101011011000, then divide modulo-2 (XOR):

1101 XOR 1011 = 0110  -> 0110011011000

1100 XOR 1011 = 0111  -> 0011111011000

1111 XOR 1011 = 0100  -> 0001001011000

1001 XOR 1011 = 0010  -> 0000010011000

(leading 0: shift)

1001 XOR 1011 = 0010  -> 0000000101000

(leading 0: shift)

1010 XOR 1011 = 0001  -> 0000000000100

(remaining leading 0s: shift)   remainder = 100

CRC bits = 100; transmitted frame = 1101011011100.

## 7.2 Received frame 1101011011101

Dividing 1101011011101 by 1011 leaves remainder 001 (not 000) → the frame is rejected – an error was detected (the last bit differs from the transmitted 100).

## 7.3 Burst errors

- The generator has degree 3, so every burst of length ≤ 3 bits is certain to be detected (confirmed by program for every position in the frame).

- Undetected pattern: any error pattern that is itself a multiple of 1011, e.g. 0000000001011 (flips the three bits 1011, a 4-bit burst) or 0000000011101 (4 bits). The corrupted frame is still divisible by G, so the receiver accepts it.

## 7.4 Two-dimensional parity (4 × 4 block)

1 0 1 1 | 1

0 1 1 0 | 0

1 1 0 0 | 0

0 1 0 1 | 0

--------+--

0 1 0 0 | 1      (row parities on the right, column parities below)

A 4-bit error that 2-D parity misses: flip the four corners of a rectangle – rows 1 and 3, columns 1 and 3. Each affected row and column has two flipped bits, so all row and column parities stay the same (verified by the program).

## 7.5 Internet checksum of 4500, 0073, 0000, 4000

Sum = 0x4500 + 0x0073 + 0x0000 + 0x4000 = 0x8573; no carry; one’s complement = 0x7A8C.

## 7.6 Complete the Code – Task07/crc.py

for i in range(len(dividend) - len(divisor) + 1):

if rem[i] == "1":

for j in range(len(divisor)):

rem[i + j] = "0" if rem[i + j] == divisor[j] else "1"

Error pattern that generator 1011 cannot detect (found with the program): 0000000001011. Reason: the receiver’s check is “remainder of frame ÷ G = 0”; when the XOR of the error pattern with the frame is divisible by G, the remainder of the error is 0, so it is invisible. Any multiple of x³ + x + 1 works.

## Alternative Concept Analysis – even parity vs CRC-3 (1011)

| Error | Even parity | CRC-3 (1011) |
| --- | --- | --- |
| 2-bit, adjacent (bits 0,1) | MISSED | caught |
| 2-bit, 7 apart (bits 0,7) | MISSED | MISSED (x⁷+1 is divisible by G) |
| 3-bit pattern 1011 (any odd number of errors) | caught | MISSED |
| 4-bit 1111 (bits 0–3) | MISSED | caught |
| 4-bit pattern 11101 (multiple of G) | MISSED | MISSED |

Even parity catches every odd-numbered error – including 3-bit pattern 1011 that CRC-3 misses – but misses every even-numbered error (all 2-bit and 4-bit patterns). CRC-3 catches all bursts ≤ 3 bits and most 2-bit and 4-bit errors that parity misses, but cannot catch some multiples of 1011. CRC is much stronger overall (and real Ethernet uses CRC-32); parity is cheap (1 bit).

hand calculations with working, an undetected-error example for each method and a comparison of their strength.

# Task 8 – Correct the Error or Ask Again?

## 8.1 Hamming check bits for a 16-bit word

2^r ≥ m + r + 1: r = 4 → 16 ≥ 21 ✗; r = 5 → 32 ≥ 22 ✓. 5 check bits. Overhead = 5/16 = 31.25 % of the data (5/21 = 23.8 % of the 21-bit codeword).

## 8.2 Minimum Hamming distance of {00000, 01011, 10101, 11110}

Pairwise distances: 3, 3, 4, 4, 3, 3 → d_min = 3. It can detect d − 1 = 2 errors and correct ⌊(d − 1)/2⌋ = 1 error.

## 8.3 Hamming(7,4) – codeword 0110011 arrives as 0110111

Syndrome = 5 → bit 5 is wrong. Flip it: 0110011; data bits (positions 3, 5, 6, 7) = 1011.

## 8.4 FEC or ARQ?

On this noisy radio link a retransmission takes 40 ms, which is long compared with the sensor’s reporting interval, so forward error correction is better when errors are frequent – the receiver repairs the reading immediately. ARQ is better when errors are rare (no overhead until needed). Video streaming and 5G use FEC because retransmitting is too slow for real-time data, there may be no return channel (broadcast/multicast), and the channel is lossy.

## 8.5 Complete the Code – Task08/hamming.py

def decode(c):

s1 = c[0] ^ c[2] ^ c[4] ^ c[6]

s2 = c[1] ^ c[2] ^ c[5] ^ c[6]

s3 = c[3] ^ c[4] ^ c[5] ^ c[6]

pos = s1 + 2 * s2 + 4 * s3

if pos:

c[pos - 1] ^= 1

return pos, [c[2], c[4], c[5], c[6]]

Two flipped bits: flipping bits 1 and 5 of the codeword gives syndrome 4, so the decoder “corrects” bit 4 – the wrong bit – and returns data 1111 instead of 1011, silently. Hamming(7,4) can correct one error but a second error cannot be detected (a SECDED extra parity bit is needed).

## Alternative Concept Analysis – BER = 10⁻³, 16-bit readings

|  | Hamming FEC | Retransmit on error (ARQ) |
| --- | --- | --- |
| Bits per reading | 21 (16 + 5 check) | 16 (+ error-detection bits not counted) |
| P(reading needs action) | P(≥ 2 errors in 21 bits) = 0.0207 % | 1 − 0.999¹⁶ = 1.588 % |
| Average transmissions | ≈ 1 | 1/(1 − 0.01588) = 1.0161 |
| Average bits sent | 21 | 16 × 1.0161 = 16.26 |
| Average extra delay | 0 ms (0.0083 ms if the rare uncorrectable ones are re-sent) | 0.0161 × 40 ms = 0.645 ms |

Choice: at this error rate ARQ uses fewer bits (16.3 vs 21) and adds an average 0.65 ms delay, but each error costs a 40 ms stall – a sensor needing predictable timing prefers FEC (fixed 21 bits, essentially no added delay). Since the plain Hamming code cannot detect double errors, use a SECDED code plus ARQ fallback.

correct calculations, a corrected codeword and a justified choice between FEC and ARQ.

# Task 9 – How Big Must the Window Be?

## 9.1 Head-office link: 400 km, 1 Gbps, 1,500-byte frames

| Quantity | Working | Result |
| --- | --- | --- |
| Transmission time | 12,000 bits ÷ 10⁹ bit/s | 12 µs |
| Propagation time | 400,000 m ÷ 2 × 10⁸ m/s | 2 ms (RTT 4 ms) |
| a = prop ÷ tx | 2 ms ÷ 12 µs | 166.67 |
| Stop-and-wait utilisation | 1 ÷ (1 + 2a) = 1 ÷ 334.33 | 0.299 % → 2.99 Mbps of 1,000 Mbps |
| Smallest window for full use | W ≥ 1 + 2a = 334.33 | W = 335 frames |
| Go-Back-N sequence numbers | W + 1 = 336 → 2⁹ = 512 | 9 bits |
| Selective Repeat sequence numbers | 2W = 670 → 2¹⁰ = 1,024 | 10 bits |

Fewer resends after one loss: Selective Repeat resends only the lost frame; Go-Back-N resends the lost frame and everything after it (up to 335 frames). Cost: the SR receiver must buffer out-of-order frames (up to 335 × 1,500 B ≈ 502.5 kB) and keep a window of its own; the GBN receiver just discards out-of-order frames (no buffer).

Frame delimiting: HDLC and PPP mark the start and end of a frame with the flag byte 01111110 (0x7E). HDLC uses bit stuffing (insert a 0 after five consecutive 1s); PPP uses byte stuffing (0x7E or 0x7D inside data is replaced by 0x7D followed by the byte XOR 0x20).

## 9.2 Complete the Code – Task09/utilisation.py

a = prop / tx

u = min(1.0, window / (1 + 2 * a))

return u, a

Satellite link, 100 % utilisation: tx = 12,000 ÷ 10⁷ = 1.2 ms; prop = 36 × 10⁶ ÷ 2 × 10⁸ = 180 ms; a = 150; W ≥ 1 + 2a = 301 frames (consistent with W = 127 giving 127/301 = 42.2 %).

## Alternative Concept Analysis – Selective Repeat with window 64

- Utilisation = 64 ÷ 334.33 = 19.14 %; throughput = 0.1914 × 1 Gbps = 191.4 Mbps.

- Receiver buffer = 64 × 1,500 B = 96,000 B ≈ 96 kB; sequence numbers needed = 2 × 64 = 128 → 7 bits.

- Window 64 is 5.2 times smaller than the 335 needed, so the link is ~80 % idle. TCP window scaling is needed on the real link: bandwidth-delay product = 1 Gbps × 4 ms = 500 kB.

correct timing, utilisation and window calculations, sequence-number sizes and a justified protocol choice (Selective Repeat).

# Task 10 – Find Where the Delay Comes From

## 10.1 Propagation delay check

400 km ÷ 2 × 10⁸ m/s = 2 ms one-way, 4 ms round trip. The traceroute RTT to hop 6 is ≈ 12.6 ms, so propagation explains 4 ÷ 12.6 = 32 %; the remaining ≈ 8.6 ms is router processing, queueing, serialisation and the fact that the real fibre route is longer than the straight line.

## 10.2 Hop 3 (38.2 ms) and hop 5 (stars)

- Hop 3, one value of 38.2 ms: traceroute measures the time of the router’s ICMP “time exceeded” reply, which the router generates on its low-priority control plane; a momentary CPU spike or ICMP rate-limit makes one probe slow. It is not a fault: the other two probes are 4.0/4.2 ms and later hops (11–12 ms) show no 38 ms delay – real forwarding delay would appear at every following hop.

- Hop 5, * * *: the router does not answer TTL-expired probes (ICMP filtered by a firewall or policy) but still forwards traffic – hop 6 replies normally. It is not a fault.

## 10.3 Which hop adds the most delay

| Hop | Median RTT | Increase over previous hop |
| --- | --- | --- |
| 1 10.10.1.1 | 0.6 ms | – |
| 2 100.64.0.1 | 3.1 ms | +2.5 ms |
| 3 172.31.4.9 | 4.2 ms (38.2 ms outlier ignored) | +1.1 ms |
| 4 103.21.8.17 | 11.8 ms | +7.6 ms (largest) |
| 6 10.200.0.10 | 12.6 ms | +0.8 ms (over hop 4) |

Hop 3 → 4 (172.31.4.9 → 103.21.8.17) adds the most delay – ≈ 7.6 ms, the jump from a private address to a public one: most probably the long-distance carrier link, which also contains the 2 ms of propagation.

## 10.4 Your own traceroute, ping and Wireshark capture

This part must be done on your own computer – the sandbox I used for the other tasks has no access to real networks, so I could not produce a genuine measurement, and the assignment asks for your own evidence. Steps:

- Windows: tracert -d <server> and ping -n 20 <server>; Linux/macOS: traceroute -n <server> and ping -c 20 <server>.

- Start Wireshark on your active interface, then visit a website. Stop and filter e.g. tcp.port == 443 || dns.

- Screenshot one packet and expand the layers: Frame (layer 1) → Ethernet II (layer 2: MACs) → Internet Protocol (layer 3: IP addresses, TTL) → TCP/UDP (layer 4: ports, flags) → Application (layer 7: DNS/HTTP/TLS) and label each header.

## Alternative Concept Analysis – monitoring methods

| Problem in the traceroute | SNMP counters (continuous) | NetFlow/flow records (continuous) | Occasional ping/traceroute |
| --- | --- | --- | --- |
| Hop 3 spike (38.2 ms) | Would not show it – counters track bytes/errors, not single ICMP delays (unless IP-SLA probes) | No | Only if run at that moment; re-runs show it was transient |
| Hop 5 stars | Not detected; not a real problem | No – traffic is seen flowing through | Shows stars but cannot tell filtering from a fault |
| Large delay on hop 3→4 link | Shows utilisation/drops/errors on the interface within the polling interval (1–5 min) | Shows which flows fill the link, within ~1–5 min of export | Shows the delay at that moment and where it starts |

Summary: SNMP + flow records give continuous history and trends (minutes) and show why a link is busy; ping/traceroute give an instant end-to-end view and locate the hop, but only when someone runs them. Best practice: combine them – continuous SNMP/NetFlow for detection, traceroute to locate the cause.

A reasoned reading of the traceroute and a propagation-delay check (done); your own measured evidence (to be inserted).

# Task 11 – Defend the Network Design

## 11.1 Recommended design (numbers from Tasks 1–10)

| Area | Medium / technology | Ethernet settings | Justification (task) |
| --- | --- | --- | --- |
| Robot cell by welders (85 m) | Cat6A S/FTP shielded, 1 Gbps (fibre if the noise worsens) | Full-duplex, auto-negotiation ON, own VLAN, strict-priority queue (DSCP EF) | T4: 85 m < 100 m; SNR 15 dB still gives 1,257 Mbps vs the 32 Mbps need; T6: avoid duplex mismatch. |
| Office floor → shop-floor switch (140 m) | Multimode fibre OM3/OM4 | Full-duplex 1 Gbps | T4: 140 m > 100 m copper limit. |
| Shop floor → data centre (550 m) | Single-mode fibre, 10 Gbps uplink | Full-duplex, jumbo frames only on the CCTV/storage VLAN | T1: 8 lines = 1,265.92 Mbps > 1 Gbps; T4: 550 m; T6: jumbo cuts overhead 2.47 → 0.42 %. |
| Plant → head office (400 km) | Leased line (carrier fibre) ≥ 1 Gbps, with backup | TCP window scaling | T4: only option at 400 km; T9/T10: 4 ms RTT, 500 kB bandwidth-delay product. |
| Offices, handhelds | Wi-Fi 6 (802.11r) | WPA3, separate VLAN | T5: fine for non-critical traffic (30 Mbps per line). |
| Sensors | Wired/Wi-Fi 6; LoRa only for slow sensors | – | T5: 600 sensors = 240 kbps > one LoRa gateway (5 kbps). |
| CCTV | Wired PoE, motion-triggered recording | Own VLAN, lowest priority | T1: 96 → 4.8 Mbps per line. |

## 11.2 Error control and flow control

- Wired links: Ethernet CRC-32 FCS on every frame (T7: CRC detects all short bursts); TCP (selective repeat/SACK) for reliable transfer; robot commands over TCP/UDP in a prioritised VLAN with no pause frames.

- WAN to head office: sliding window with W ≥ 335 frames needed for full use (T9) – 10-bit sequence numbers for Selective Repeat; stop-and-wait would give only 2.99 Mbps.

- Wireless sensor links: forward error correction (T8: Hamming adds 5 check bits = 31.25 % but avoids a 40 ms retransmission); upgrade to SECDED codes since plain Hamming miscorrects double errors.

## 11.3 The two biggest remaining risks

- Congestion or failure of the shared uplink: at 126.6 % load (T1) robot commands queue behind video → mitigated by 10 Gbps, QoS priority and a redundant ring/second uplink (recovery ≈ tens of ms, T6).

- Wireless mobility and noise: Wi-Fi hand-overs of 150 ms (40 per trip = 6 s unreachable, T5) and welding interference cannot meet 10 ms – mitigated by keeping robots wired and (Task 12) moving mobile equipment to private 5G.

## 11.4 Does it work for five plants?

Per plant (8 lines) the traffic is 1,265.92 Mbps; five plants = 6,330 Mbps in total, but robot control (256 Mbps per plant) and CCTV (768 Mbps per plant, 38 Mbps with motion triggering) stay local. Only office traffic (30 Mbps × 8 = 240 Mbps) and sensor data (1.92 Mbps) cross the WAN: ≈ 242 Mbps per plant, ≈ 1.21 Gbps for five plants worst case – workable with a ≥ 1 Gbps leased line per plant and a 10 Gbps head-office aggregation. Use VLANs per line (≈ 40 devices per /26) to keep broadcast domains small as the plant grows to 1,500 devices.

## Alternative Concept Analysis – everything on private 5G instead of wired backbone + Wi-Fi

| Traffic (8 lines) | Rate | Carry on 5G? | Reason |
| --- | --- | --- | --- |
| Robot control | 256 Mbps | Fixed robots: cable; mobile/reconfigurable ones: 5G URLLC | Deterministic, EMI-free on cable; 5G URLLC is for equipment that moves. |
| AGVs | 40 Mbps (50 AGVs) | 5G (URLLC) | 4 hand-overs × 20 ms vs Wi-Fi 6 s per trip. |
| Sensors | 1.92 Mbps | 5G (mMTC) | Many small devices, tiny load. |
| Office | 240 Mbps | Wi-Fi 6 / cable | Cheap capacity, not latency-critical. |
| CCTV | 768 Mbps | Cable (fibre/PoE) | A 5G cell shares a few hundred Mbps (assumed); 768 Mbps video would starve control traffic. |

A recommendation supported by calculations from Tasks 1–10 (measured evidence from Task 10.4 to be added by you).

# Task 12 – Re-evaluate and Adapt the Design

Decision: MODIFY – keep the wired backbone, fibre/shielded cabling, error-control settings and Wi-Fi 6 for offices; move AGVs and mobile equipment to a private 5G URLLC network and upgrade the uplink to 10 Gbps.

## A. Impact analysis (2 marks)

| Affected element | Impact of private 5G + 50 AGVs |
| --- | --- |
| Wireless choice | AGVs cannot rely on Wi-Fi 6 – 40 hand-overs of 150 ms each per 1.2 km trip (T5). |
| Media | 5G radio units/small cells need fibre backhaul (single-mode, 550 m run already planned) and PoE/power; the 1 Gbps uplink must grow. |
| Error control | 5G uses LDPC/polar coding with HARQ (FEC + fast retransmission) on the air link; wired CRC-32/TCP unchanged. |
| Traffic | See table below. |

Assumption (AGV traffic was not given): each AGV sends control/telemetry like a robot controller – 200 bytes every 2 ms = 0.8 Mbps; optional on-board camera 4 Mbps.

| Scenario | Calculation | Plant traffic | Load on 1 Gbps / 10 Gbps |
| --- | --- | --- | --- |
| Original (8 lines) | 8 × 158.24 | 1,265.92 Mbps | 126.6 % / 12.7 % |
| + 50 AGVs, control only | 1,265.92 + 50 × 0.8 | 1,305.92 Mbps | 130.6 % / 13.1 % |
| + 50 AGVs with cameras | 1,305.92 + 50 × 4 | 1,505.92 Mbps | 150.6 % / 15.1 % |

## B. Test the original design

| Test | Original design (Wi-Fi 6 for mobile) | 5G |
| --- | --- | --- |
| Hand-overs per AGV trip (1.2 km at 2 m/s = 600 s) | 40 | 4 |
| Unreachable time per trip | 40 × 150 ms = 6.0 s (1.0 % of time) | 4 × 20 ms = 80 ms (0.013 %) |
| With 802.11r (~50 ms) | 40 × 50 ms = 2.0 s | – |
| Hand-overs for 50 AGVs per cycle | 2,000 (3.3 per second across the fleet) | 200 |
| 2-ms commands lost per hand-over | 75 | 10 |
| Meets 10 ms deadline during hand-over? | No – 150 ms is 15× the deadline | Not quite (20 ms) – needs dual connectivity/make-before-break |

Result: the original design cannot serve 50 moving AGVs – every hand-over exceeds the 10 ms deadline by 15× and happens 3.3 times per second somewhere in the fleet. 5G cuts blind time 75-fold (6 s → 80 ms per trip). Also, the 1 Gbps uplink is already overloaded (126.6 %) and goes to 130.6 % with AGVs.

## C. Adaptation or redesign

- Private 5G (URLLC slice): AGVs, mobile robots, handhelds; overlapping cells of 300 m (4 cells on the 1.2 km route) with dual connectivity so the AGV stays attached during hand-over; AGVs must also stop safely on loss of command.

- Wi-Fi 6: offices only (30 Mbps per line); mMTC / LoRa: sensors.

- Fibre backbone: 10 Gbps single-mode uplink (13.1 % loaded with AGVs); backup uplink in a ring; 5G core on-premises so commands never leave the plant.

- Unchanged: shielded Cat6A in the robot cell, multimode fibre for the 140 m run, leased line for the 400 km head-office link, CRC-32/TCP, QoS priority for robot and AGV control.

## D. Evidence-based defence (2 marks)

- Why not RETAIN: Wi-Fi gives 6 s unreachable per trip against the “never miss a command” rule; the uplink is at 130.6 %.

- Why not REDESIGN completely: the wired design is sound – 5G cannot carry 768 Mbps of CCTV plus 256 Mbps of robot control, and cable is immune to RF problems near the welders; the original numbers (robot cell 32 Mbps within 1,257 Mbps capacity at 15 dB; 85 m < 100 m) still hold. Rebuilding would add cost without benefit.

- Why MODIFY works: 5G reduces hand-over time 75× (6 s → 80 ms per trip) and the AGVs’ 40 Mbps is a tiny load on a 10 Gbps backbone (13.1 %).

- Trade-offs: extra cost of the 5G core, radios and spectrum/licence; 5G’s 20 ms hand-over still needs mitigation; two technologies to operate (5G + Wi-Fi) – accepted because each is used where it is strongest.

A quantified impact analysis, a calculation-based test of the original design, a clear modify-decision and a defence with numbers and trade-offs.